Two-Stage CMOS Operational Amplifier

Design, Sizing, and LTspice Simulation Report

# 1. Objective

This project designs and verifies a two-stage CMOS operational amplifier in LTspice, built around a 5-transistor differential input stage with a current-mirror active load, followed by a common-source second gain stage. A Miller compensation network (compensation capacitor plus a nulling resistor) is used to guarantee frequency stability, and the design is validated through DC operating point, AC, and transient simulations.

# 2. Circuit Topology

The amplifier consists of two stages:

- Stage 1 - Differential input stage: M1/M2 (NMOS input pair) with M3/M4 (PMOS current-mirror active load) and M5 (NMOS tail current source, biased by Vbias).

- Stage 2 - Common-source gain stage: M6 (PMOS) as the gain device, M7 (NMOS current source) as its load, with its gate tied to the same Vbias net as M5.

- Compensation network: a Miller capacitor Cc in series with a nulling resistor Rz, connected from the output (Vout2) back to the stage-1 output / stage-2 input node (Vout1).

Figure 1: Final schematic with transistor sizing, compensation network, and load capacitance.

# 3. Design Equations and Transistor Sizing

All devices were designed for an overdrive voltage of Vov = 0.2V, corresponding to gm/ID = 2/Vov = 10 V⁻¹, using the LTspice square-law models below:

- .model NMOS_MY NMOS (VTO=0.7 KP=110u LAMBDA=0.05)

- .model PMOS_MY PMOS (VTO=-0.7 KP=50u LAMBDA=0.06)

Design targets: GBW = 5MHz, Cc = 1pF, CL = 2pF, gm6/gm1 = 8 (stage-2/stage-1 transconductance ratio), IB2/IB1 = 4.

| Parameter | Value |
| --- | --- |
| gm1 (M1, M2) | 31.4 uA/V |
| gm6 (M6) | 251 uA/V |
| ID1 = ID2 | 3.14 uA |
| IB1 = ID(M5) | 6.28 uA |
| ID6 = ID(M7) | 25.1 uA |

Resulting transistor sizes (L = 1um for all devices):

| Device | Role | ID | W/L | W (L=1um) |
| --- | --- | --- | --- | --- |
| M1, M2 | NMOS input pair | 3.14 uA | 1.4 | 1.4 um |
| M3, M4 | PMOS mirror load | 3.14 uA | 3.1 | 3.1 um |
| M5 | NMOS tail source | 6.28 uA | 2.9 | 2.9 um |
| M6 | PMOS gain stage (stage 2) | 25.1 uA | 25.1 | 25 um |
| M7 | NMOS current source load | 25.1 uA | 11.4 | 11.4 um |

Bias network: Vbias = Vt + Vov = 0.9V, applied to both M5 and M7 (identical Vov, current ratio set by W/L). Input common-mode range derived from saturation constraints on M1, M3, and M5:

- Lower bound: Vicm ≥ Vov5 + Vt1 + Vov1 = 1.1V

- Upper bound: Vicm ≤ VDD - 0.2V

For the test supply VDD = 3.3V, this gives a valid range of 1.1V - 3.1V; Vicm = 2.1V was used for all simulations, comfortably within range.

# 4. Miller Compensation and Nulling Resistor

Without a nulling resistor, the Miller capacitor introduces a right-half-plane (RHP) zero at z = gm6/Cc, which erodes phase margin. A series resistor Rz is added with Cc, and choosing Rz = 1/gm6 pushes this zero out to infinity, removing its effect on phase margin entirely.

Rz = 1/gm6 = 1/251uA/V ≈ 4kΩ (used in the design).

# 5. Simulation Results

## 5.1 DC Operating Point (.op)

All seven transistors were confirmed in saturation (Vds > Vov) at VDD = 3.3V, Vbias = 0.9V, Vicm = 2.1V. Simulated bias currents were consistently 7.6-9.8% above hand-calculated values, and the IB2/IB1 ratio came out to 4.08 versus the 4.0 target. Both effects are attributable to the (1 + LAMBDA * Vds) channel-length-modulation term included in the SPICE model but omitted from the first-order square-law hand calculation.

## 5.2 AC Analysis (.ac)

Figure 2: Open-loop AC response (magnitude and phase) with cursor readings at DC and at the 0dB crossover.

DC gain, GBW, and phase margin were extracted directly from the Bode plot cursors.

## 5.3 Transient Analysis (.tran) - Slew Rate

The amplifier was reconfigured as a unity-gain buffer (Vout2 fed back to the inverting input) and driven with a 0.5V step (2.1V to 2.6V) to force slew-rate-limited operation. The output shows the expected two-phase response: a straight-line slewing segment followed by exponential settling.

Figure 3: Closed-loop step response showing slew-rate-limited rise followed by settling; cursor slope gives the measured slew rate.

Theoretical slew rate: SR = IB1/Cc = 6.28uA / 1pF = 6.28 V/us, derived from the fact that during a large step, one input transistor shuts off entirely and the other carries the full tail current IB1 to charge Cc.

# 6. Hand-Calculation vs. Simulation Summary

| Parameter | Hand Calculation | Simulated (LTspice) | Deviation |
| --- | --- | --- | --- |
| DC Gain | 78 dB | 79.4 dB | +1.4 dB |
| Gain-Bandwidth Product | 5 MHz | 5.00 MHz | ~0% |
| Phase Margin | 76° (90° - arctan(GBW/p2)) | 77.8° | +1.8° |
| Slew Rate | 6.28 V/us | 6.425 V/us | +2.3% |

Phase margin hand calculation uses the two-pole approximation PM = 90° - arctan(GBW/p2), with the non-dominant pole estimated as p2 ≈ 4 x GBW = 20MHz for a well-compensated two-stage amplifier with the RHP zero cancelled by Rz.

# 7. Conclusion

The two-stage CMOS operational amplifier meets all design targets, with simulated results tracking hand calculations to within 10% across gain, GBW, phase margin, and slew rate - the largest deviations traceable directly to the channel-length-modulation term (LAMBDA) omitted from the first-order square-law sizing equations. The design achieves a phase margin of 77.8°, comfortably above the 60° stability threshold, confirming the nulling resistor successfully cancels the RHP zero introduced by Miller compensation.